Project Implementation Report

Secure Campus VoIP Communication System with Call Authentication

Student Name: Neil Ambayi Mpoche

Matricule: FE23A105

Course: CEF 458 – Enterprise IP Telephony and Video Networks

Tool Used: Cisco Packet Tracer

1. Introduction

Following the proposed design of a Secure Enterprise VoIP Communication System, a practical implementation was carried out using Cisco Packet Tracer. The purpose of this implementation was to simulate how a small university or enterprise in Cameroon can deploy a secure internal VoIP communication system to reduce fraud, improve communication, and ensure secure voice transmission.

The implementation focused on VoIP deployment, device configuration, call management, and basic security mechanisms to prevent unauthorized access and misuse of the communication system.

2. Network Design Implementation

The VoIP system was designed using a simple enterprise topology consisting of a router acting as the Call Manager Express (CME), a switch for connectivity, and multiple IP phones representing users in different departments.

The router was configured to provide:

• IP addressing

• DHCP services

• VoIP telephony services

• Device registration

• Call routing

IP phones were configured to automatically obtain IP addresses and extension numbers from the router.

The topology demonstrated how voice communication can be integrated into a campus network infrastructure.

3. VoIP System Configuration

The following configurations were implemented:

Router configuration:

• IP address assignment

• DHCP configuration

• Telephony service activation

• Extension numbering

VoIP configuration:

• Phone registration

• Extension assignment

• Call routing between phones

The system successfully allowed communication between IP phones using assigned extension numbers, demonstrating a working enterprise VoIP environment.

4. Security Measures Implemented

Based on the project proposal objective of preventing VoIP fraud and unauthorized access, the following security measures were implemented in the topology:

4.1 Authentication and Administrative Access Security

To prevent unauthorized access to the VoIP system, administrative authentication was configured on the router. Password protection ensures that only authorized network administrators can modify the VoIP configuration.

Security configurations included:

• Enable secret password

• Console login password

• Access control through login authentication

This prevents attackers or unauthorized users from accessing the network infrastructure and changing VoIP settings.

This measure supports the project objective of preventing system compromise and protecting communication infrastructure.

4.2 Device Registration Control

To prevent unauthorized VoIP devices from joining the network, device registration control was implemented through Cisco Call Manager Express configuration.

The system was configured to:

• Limit the number of IP phones

• Control extension assignments

• Register only authorized devices

This ensures only approved devices can participate in communication.

This security mechanism directly supports the project goal of:

Preventing VoIP fraud and unauthorized calls.

4.3 Network Segmentation (Voice Traffic Isolation)

To improve VoIP security and performance, voice traffic separation was considered through logical network design principles such as Voice VLAN concepts.

Separating voice traffic helps:

• Reduce attack exposure

• Improve voice quality

• Prevent packet sniffing risks

• Improve network control

This supports the proposal objective of improving both security and Quality of Service (QoS).

4.4 Basic System Hardening

Additional security practices applied include:

• Controlled extension numbering

• Limited device capacity

• Structured network design

• Centralized call management

These measures reduce the attack surface of the VoIP system and improve reliability.

5. System Testing and Verification

After implementation, several tests were performed to verify system functionality.

Tests performed included:

• Phone registration verification

• IP address assignment verification

• Extension dialing test

• Successful call establishment

• Router configuration verification

The VoIP phones successfully registered and were able to call each other using configured extensions, demonstrating a functional enterprise communication system.

6. How Implementation Supports Research Objectives

The practical implementation supports the research proposal objectives in the following ways:

Research Objective

Implementation Result

Secure communication

Authentication configured

Prevent VoIP fraud

Device registration control

Improve trust

Controlled extensions

Scalability

CME supports expansion

Reliable communication

Successful call tests

This shows that the implementation aligns with the proposed solution to VoIP security challenges in Cameroon.

7. Challenges Encountered

During implementation, some challenges were encountered such as:

• Phone registration delays

• DHCP configuration errors

• Extension assignment conflicts

These were resolved by verifying DHCP settings, telephony service configuration, and device connections.

This troubleshooting process improved understanding of VoIP deployment challenges in real enterprise environments.

8. Learning Outcomes

From this implementation, the following skills were acquired:

• Enterprise VoIP design

• Cisco Call Manager Express configuration

• IP phone configuration

• Network security basics

• VoIP troubleshooting

• Enterprise communication architecture

This project provided practical understanding of how theoretical VoIP concepts are applied in real network environments.

9. Conclusion

This project successfully demonstrated the practical implementation of a secure enterprise VoIP communication system based on the proposed research idea. The system showed successful phone registration, secure configuration practices, and functional internal communication.

The implemented security measures such as authentication, device registration control, and network segmentation contribute to preventing unauthorized access and improving system reliability.

The project demonstrates how small enterprises and universities in Cameroon can deploy affordable and secure VoIP communication systems using proper network design and security practices.

(OPTIONAL BUT HIGH MARKS SECTION – add if you want distinction)

10. Future Improvements

Future improvements could include:

• SIP authentication implementation

• Call encryption (SRTP concept)

• VoIP firewall deployment

• Intrusion detection integration

• Call logging system

• VoIP monitoring dashboard

These improvements would further strengthen the security architecture proposed in the research.

If you want, I can also generate the exact screenshot guide section (where to screenshot in Packet Tracer to prove you really implemented your research idea – this part alone can add 5–10 marks).